Intra block copy

IBC merge/AMVP list construction

Three candidates are located on (the( (nearest) corners) of the reference (region, (and)) three) additional candidates are determined in the middle of the three sub-regions (A, B, and C), whose coordinates are determined by the width, and height of the current block and the ΔX and ΔY parameters, as is depicted in Figure 64.

for( I =0+1-4; )

( 12 3)

for( I =0; sh_slice_type = = I && cu_skip_flag[ x0 ][ y0 ] = =0 ; I ++)

if( ( ( sh_slice_type  = =  I  &&  cu_skip_flag[ x0 ][ y0 ] = =0 )  | |
		( sh_slice_type  !=  I  &&  ( CuPredMode[ chType ][ x0 ][ y0 ] !=  MODE_INTRA  | |
		( ( ( cbWidth  = =  4  &&  cbHeight  = =  4 )  | |  modeType  = = MODE_TYPE_INTRA )
		&&  cu_skip_flag[ x0 ][ y0 ]  = =  0 ) ) ) )  &&
		cbWidth  <=  64  &&  cbHeight  <= 64  &&  modeType  !=  MODE_TYPE_INTER  &&
		sps_ibc_enabled_flag  &&  treeType  !=  DUAL_TREE_CHROMA )

Figure 62

BV with a zero-horizontal component.

Figure 64. Padding candidates for the replacement of the zero-vector in the IBC list.

Radius=log2((cbWidth∙cbHeight)≫MIN_PU_SIZE)

Figure 65. IBC candidate clustering based on the L2 distance and the TM cost.

The optimal IBC AMVP index is signalled, which allows deriving the sign of the non-zero BVD component at the decoder side. The absolute magnitude of non-zero BVD component is further signalled. In RRIBC, the direction of the flipping mode is derived from the signalled directional flag.

IBC with Template Matching

Template Matching is used in IBC for both IBC merge mode and IBC AMVP mode.

The IBC-TM merge list is modified compared to the one used by regular IBC merge mode such that the candidates are selected according to a pruning method with a motion distance between the candidates as in the regular TM merge mode. The ending zero motion fulfillment is replaced by motion vectors to the left (-W, 0), top (0, -H) and top-left (-W, -H), where W is the width and H the height of the current CU.

Figure 66: IBC reference region depending on current CU position.

IBC reference area

The reference area for IBC is extended to two CTU rows above. Figure 67 illustrates the reference area for coding CTU (m, n). Specifically, for CTU (m, n) to be coded, the reference area includes CTUs with index (m–2, n–2)…(W, n–2),(0, n–1)…(W, n–1),(0, n)…(m, n), where W denotes the maximum horizontal index within the current tile, slice or picture. When CTU size is 256, the reference area is limited to one CTU row above. This setting ensures that for CTU size being 128 or 256, IBC does not require extra memory in the current ETM platform. The per-sample block vector search (or called local search) range is limited to [(C << 1), C >> 2] horizontally and [–C, C >> 2] vertically to adapt to the reference area extension, where C denotes the CTU size.

Figure 67. Reference area for IBC when CTU (m, n) is coded. The blue block denotes the current CTU; green blocks denote the reference area; and the white blocks denote invalid reference area.

Fractional pel IBC

The option of block vector resolutions is extended to include quarter-pel resolution in additional to full-pel and 4-pel. Like inter AMVR syntax, the first bin is signalled to indicate whether BV is in quarter-pel resolution, and the second bin is signalled to switch between full-pel and 4-pel resolutions. The interpolation filters applied to the luma (8-tap) and chroma (6-tap existed inter interpolation) components of an IBC block. For template-based IBC tools, a 2-tap bilinear interpolation filter is applied to generate template prediction blocks. Reference sample padding is performed when some of them are located outside IBC reference area. When needed, it performs in horizontal direction first and then vertical direction.

Filtered IBC prediction

Additional filtered IBC mode is introduced, where a filter is applied to IBC predictor, which is derived by minimizing MSE between current and reference template.

Output of the filter is calculated as follows:

TEST:

abc_def / abc_def_ghi / a0_def_ghi /

log2CbSizeC / log2TrafoSize / xCb, nCbSX, xTbCmp, cIdx /

xY, qP, bS,

x0, y1

d,

CbSaX

ABC_MODE

If / if / When / when / coding unit

Cb is greater than 10 / Cb is less than 10 / A is equal to log2TrafoSize[ 0 ][ 1 ] / Cb are not equal to log

Cbs are set equal to ABC_MODE / Cbs are set equal to abc_def

ue(v), se(v), or ae(v) ae(v), Max( 0, ((Qstate) − 1))

수학식: fDk,l=fl,k, cDk,l=cl,k / 대응텍스트: ,𝑓-𝐷.,𝑘,𝑙.=𝑓,𝑙,𝑘., ,𝑐-𝐷.,𝑘,𝑙.=𝑐,𝑙,𝑘.

Kx,y=min⁡(y,max-y,x)

predLumaVal = c0C + c1N + c2S + c3E + c4W + c5P + c6B

The nonlinear term P is represented as power of two of the center sample C and scaled to the sample value range of the content:

P = ( C*C + midVal ) >> bitDepth

The bias term B represents a scalar offset between the input and output and is set to middle luma value (512 for 10-bit content).

This filtered mode is used as an additional mode for non-merge IBC blocks, and it is not used together with IBC-LIC, IBC-CIIP or RR-IBC. For IBC merge modes, this filtering mode is inherited when merge mode list is constructed. The mode flag is signalled before the IBC-LIC flag.

BVD prediction

Similar to MVD prediction, possible BVD sign combinations of IBC mode are sorted according to the template matching cost. Moreover, the first 4 most signification suffix bins of exponential Golomb code used to represent BVD magnitudes is also sorted according to the TM cost. An example is shown in Figure 68. Template matching operation is used to determine a BVD candidate with the best cost and indicate in the bitstream whether the best candidate is predicted correctly or not.

Figure 68. Prediction of BVD.

Reconstruction-Reordered IBC (RR-IBC)

A Reconstruction-Reordered IBC (RR-IBC) mode is allowed for IBC coded blocks. When RR-IBC is applied, the samples in a reconstruction block are flipped according to a flip type of the current block. At the encoder side, the original block is flipped before motion search and residual calculation, while the prediction block is derived without flipping. At the decoder side, the reconstruction block is flipped back to restore the original block.

Two flip methods, horizontal flip and vertical flip, are supported for RR-IBC coded blocks. A syntax flag is firstly signalled for an IBC AMVP coded block, indicating whether the reconstruction is flipped, and if it is flipped, another flag is further signaled specifying the flip type. For IBC merge and BV candidates in SGPM mode, the flip type is inherited from neighbouring blocks, without syntax signalling. Considering the horizontal or vertical symmetry, the current block and the reference block are normally aligned horizontally or vertically. Therefore, when a horizontal flip is applied, the vertical component of the BV is not signaled and inferred to be equal to 0. Similarly, the horizontal component of the BV is not signaled and inferred to be equal to 0 when a vertical flip is applied.

To better utilize the symmetry property, a flip-aware BV adjustment approach is applied to refine the block vector candidate. For example, as shown in Figure 69, (xnbr, ynbr) and (xcur, ycur) represent the coordinates of the center sample of the neighbouring block and the current block, respectively, BVnbr and BVcur denotes the BV of the neighbouring block and the current block, respectively. Instead of directly inheriting the BV from a neighbouring block, the horizontal component of BVcur is calculated by adding a motion shift to the horizontal component of BVnbr (denoted as BVnbrh) in case that the neighbouring block is coded with a horizontal flip, i.e., BVcurh =2(xnbr -xcur) + BVnbrh . Similarly, the vertical component of BVcur is calculated by adding a motion shift to the vertical component of BVnbr (denoted as BVnbrv) in case that the neighbouring block is coded with a vertical flip, i.e., BVcurv =2(ynbr -ycur) + BVnbrv .

Figure 69. Illustration of BV adjustment for (a) horizontal flip, and (b) vertical flip, respectively.

(a)

(b)